| Ref. | Additional Notes | Recommendations | Priority |
| --- | --- | --- | --- |
| T3508 | 3m long dead stub from previous pollarding of a primary limb beneath the phonelines Decay pocket at 1m on main stem - probed to 500mm, tap tested - hollow. | Brace stems at 10m and reduce tree all over by 2.5-3m to reduce forces acting on cavity. | 6 Months |
| G3532 | Group of 4 trees. Significant dieback of northernmost 3 trees. | Remove northern 3 trees owing to lack of suture potential and risk of failure. | 12 Months |
| T3501 | Moderate deadwood. Branch tips low | Remove significant deadwood. Crown lift to 3m (tertiary branches only). | IBA |
| T3503 | Branch tips low. | Target prune lowest whirl of limbs back to main stem to raise crown to 2m . | IBA |
| T3504 | Dieback of branch tips - likely from inadequate irrigation. | Ensure irrigation during summer months. Prune out deadwood. | IBA |
| T3505 | Dumped green waste over rooting zone. min historic cambial damage to buttress roots - not of concern. | Remove green waste to prevent root issues. | IBA |
| T3506 | Branch tips low. Damage to base of stem, cavity and decay evident - tap tested and probed - no safety concerns at present. Late into leaf - not of concern. | Crown lift to 3m (tertiary branches only). | IBA |
| T3507 | Branch tips low over path. Moderate historic cambial damage to base - not of concern. | Crown lift to 3m (tertiary branches only). | IBA |
| T3512 | Low sweeping branches - not of concern. Seating area beneath tree. Lateral tree branches heavily weighted. Tree may benefit from 1m all over crown reduction to reduce risk of branch failure in strong winds. If done this must be done sympathetically to the trees age and aesthetic, and with the purpose of reducing the branch end weight of heavily loaded branches. Power lines running through crown. | Tree may benefit from 1m all over crown reduction to reduce risk of branch failure in strong winds. If done this must be done sympathetically to the trees age and aesthetic, and with the purpose of reducing the branch end weight of heavily loaded branches. Power lines running through crown. (Consider within 5 years). | IBA |
| T3513 | Branch tips low over green. | Crown lift to 2m (tertiary branches only). | IBA |
| T3514 | Uncertain of species. Support stakes and ties still in place. | Bark mulch should be applied to a depth of 50mm around the base of the tree forming a 1m circle around the tree's stem. The mulch should not be heaped up around the stem of the tree. | IBA |
| T3523 | Support stakes and ties still in place. | Bark mulch should be applied to a depth of 50mm around the base of the tree forming a 1m circle around the tree's stem. The mulch should not be heaped up around the stem of the tree. Irrigate during summer months. | IBA |
| T3525 | Moderate deadwood in crown. Long heavily weighted branches near to carpark ng spaces. | Remove significant deadwood. Resident requests that this tree is reduced - could be reduced by 5m all over. it would be advisable to do this within 5 years . | IBA |
| T3526 | Moderate deadwood over pond. Remove lowest branch extending over bench owing to defect above bench. | Remove lowest branch extending over bench back to the stem. | IBA |
| T3529 | Minor deadwood within crown. Strong winds may result in branch breakages. | Remove significant deadwood. Reduce tree by ~5m all over (within 5 years). | IBA |
| T3531 | Minor deadwood within crown. Strong winds may result in branch breakages. | Reduce tree by ~5m all over (within 5 years). | IBA |
| T3534 | Tree in neighbouring ownership with a heavy lean and deadwood over the site. | Remove deadwood overhanging the site, and request tree owner to reduce the height of the tree to approx. 8m on the main stem, to remove future contention and safety issues | IBA |
| T3535 | Moderate deadwood in crown. Strong winds may result in branch breakages. Powerlines are present beneath crown. | Remove significant deadwood. Reduce tree by ~5m all over (within 5 years). | IBA |
| G3522 | Mixed species group. Moderate deadwood in crown. One standing dead stem. | Remove dead stem and deadwood from crown. | IBA |